报告题目：Improved gas sensing using three-dimensional graphene

摘要：

Graphene (Gr) based gas sensors have attracted widespread research interest because of their potential use in the next generation technologies such as Internet of Things (IoT). Recently, 3D Gr porous structures demonstrate improved gas sensing performance compared with the 2D counterparts due to their unique porous structure with enlarged surface area. We demonstrate a better gas sensing system with 3D Gr synthesized by several different strategies. The 3D reduced graphene oxide hydrogel (RGOH) synthesized by a one-step, hydrothermal, and self-assembly method can be exploited for high-performance NO2 and NH3 sensing. Importantly, an integrated microheater is exploited for the first time to improve the selectivity of NO2 sensing. Compared with an unmodified RGOH counterpart, the chemically functionalized reduced graphene oxide hydrogel (FRGOH) not only displays twofold higher sensitivity in both NO2 and CO2 sensing, but also exhibits significantly faster recovery and a lower limit of detection. The microwave plasma-enhanced chemical vapor deposition (MPCVD) is deployed to synthesize cauliflower Gr for NO2 detection with high performance, including ultrahigh sensitivity (133.2 ppm-1), good selectivity, fast recovery (within 2 s) and reversibility. In addition, 3D spark plasma sintering (SPS)-synthesized, superhydrophobic and hierarchical reduced graphene oxide is exploited for high-performance NO2 detection with good immunity to humidity.

汇报人简介：

吴进，中山大学“百人计划”副教授，博士生导师。在华中科技大学和新加坡南洋理工大学分别获得硕士和博士学位。2014年博士毕业后，在南洋理工大学从事新加坡-麻省理工学院联盟(SMART)项目里面的气体传感器的研究。 2017年加入中山大学电子与信息工程学院。主要研究方向为气敏/湿敏传感材料和器件，可穿戴式电子（湿度、温度、气体、应变、压力等传感器件），高精度纳米印刷和微纳加工技术。他已发表SCI论文接近30篇，其中第一作者14篇，包括Advanced Functional Materials，Advanced Science, Nanoscale，Journal of Materials Chemistry A，ACS applied materials & interfaces等杂志。